ECE 5140/6140

- Design a 3-stage amplifier with a gain of -2000 for a signal that has a bandwidth of 3 MHz. Assume that the amplifier you must use has a GBW of 165 MHz.

- A non-inverting amplifier with a gain of 130 V/V has a voltage offset (VOS) of 150mV, what is the DC value at the output of the amplifier?

- A non-inverting amplifier with a gain of 750 is connected to a resistor of 15kOhm. If the input bias current (IB) is 10nA, what is the resulting DC offset at the output of the amplifier?

- An inverting amplifier with a gain of 120 is connected to a sensor that has a resistance of 15kOhm. The feedback resistor, Rf, is equal to 25kOhms. Assume that the sensor has no DC voltage for this calculation.
a) What is Rin? 

b) If the input bias current (IB) is 15nA, what is the resulting DC offset at the output of the amplifier?

- A non-inverting amplifier with a gain of 750 is connected to a resistive sensor with an R0 of 5kOhm. The span of the sensor is 2kOhm. The amplifier has an input bias current (IB) of 4.5nA and a voltage offset (VOS) of 12mV.
a) What is the range of DC offsets?

b) If the amplifier has a maximum supply voltage of ±10V and requires a voltage overhead of 1.5V on each rail, what is the maximum peak-to-peak voltage of the input signal from the sensor?

- Do problem 11.3 in the text

- Do problem 11.38 in the text

- A strain gauge is placed into a quarter bridge and has R0 = 3.3 kOhm @ 20o C and has a sensitivity of 100 Ohm/C. Assume the bridge circuit uses a 5V reference.
a) Identify the range of voltages that are approximately linear such that the maximum error is 15%. Calculate error as 
                                                     PercentError = |(Expected – Actual)/Actual|
b) Design two amplifier chains such that the output uses 70% OFS of a rail-to-rail amplifier using a ±15V supply. For each design, pick one of the 5 architectures shown in lecture. No stage should have a gain more than 40V/V. Simulate the circuit to demonstrate that it works. 

c) ECE 6140: First, justify the amplifier you chose to use in the design, then discuss design tradeoffs between the two architectures you picked.